AKTEK

White Cement Based Hand Plaster

White view on the sides of the buildings.

DEFINITION

Cement Based Ready Mix Hand Plaster for external and internal parts of the buildings

CHARACTERISTICS

- It is easy and fast to apply.

- Provides a strong surface with high adhesive resistance

# APPLICATION FIELDS

Applied on  brick , gas concrete , concrete,  fair faced concrete, bims , briquette walls and ceilings of interior and exterior  faces of the buildings.

STORAGE & PACKING

- 50 kg kraft paper or polypropylene bags, in the sling or shrink covered pallets upon request.

# 6 months from the date of manufacture if stored in unopened original package; under dry, frost free conditions and arranged maximum 8 bags one above the other. Suggested to be consumed in 1 week after the package is opened .

SPECIAL SUGGESTIONS

- The surface must be purified from form oil, dust, paint and adherence reducing materials.

- All the holes must be filled and repairs must be made with appropriate Entegre product before plastering.

- Application surface must be dampened with water.

- Fiber mesh must be used at the intersection of different materials and at points, where some movements can occur.

- PRİMEL® (Primer) must be applied on fair faced and walls and ceilings at least 72 hours before AKTEK®.

- For the application on AKTEK ®, you must be sure that the surface is not damp.

- Perfect adaptation with AKFİN.

- No other materials (lime, cement, gypsum etc..) should be added to the prepared mortar

- Suggested that the application is done between +5°C, +35°C.

- AKTEK  ® should be protected against quick drying in windy weather conditions.

- Don’t go beyond the application field and rules.

- As it is cement based product, do not smell it directly.  Wash your eyes in case of contact. Call your doctor if it is necessary.

# For detailed information please request security data sheet.

# Contact  us for the other application fields and for your technical questions.

## APPLICATION RULES

- AKTEK®  is mixed with ready mix plaster mixing and spreading machine (10,5-11,5 lt. water / 50 kg bag) and sprayed to the surface.

- Application thickness must be between 1 cm and 2,5 cm in one coat .If the thickness is more than 2.5 cm, a second coat is applied in max. 2.5 cm thickness after one day.

- Surface is leveled.

- AKTEKK® is polished when the surface becomes hard enough and if  necessary the operation is completed by a dampened sponge.

# CURE

For the durability of plaster, in 7 days after the application, depending on weather conditions, the surface must be dampened at intervals

CONSUMPTION

For 1 cm plaster thickness, the consumption is approximately 13 kg/m².

#### TECHNICAL DATA

| # Dry bulk density, kg/m³ | 1200 ± 150 |
| --- | --- |
| Wet bulk density, kg/m3 | 1600 ± 200 |
| ### Compressive Strength, N/mm² | > 2,5 |
| Flexural Strength, N/mm² | > 1 |
| Adhesion, N/ mm² | > 0,3 |

REFERANCE CERTIFICATE

TSE certificate.